EE 586 Signal and Power Integrity

Project 2 Report

Fall 2024

Submitted by:

Dagmawi Abera

### 1. HFSS Design

The first image shows the ANSYS HFSS (High-Frequency Structure Simulator) design of two closely spaced microstrip lines, used for analyzing signal propagation and crosstalk (NEXT and FEXT).

#### Key Design Components:

- Substrate: The green base material acts as the dielectric medium for the microstrip lines, typically FR4.

- Two Microstrip Lines:

  - Orange Line: Represents one of the signal traces.

  - Blue Line: Represents the other signal trace placed close to the first line to introduce crosstalk effects.

- Ports: The ports are placed at the start and end of each line:

  - Port 1 and Port 3: Input ports for the signal injection.

  - Port 2 and Port 4: Output ports for analyzing the signal's behavior, including S-parameters.

The objective of this design is to analyze the crosstalk between the two lines using S-parameters, where:

- S11​ and S22​: Input reflection coefficients.

- S21​ and S12​: Transmission coefficients, representing Near-End Crosstalk (NEXT) and Far-End Crosstalk (FEXT).

### 2. S-Parameter Results

The second image shows the S-parameter results obtained from the HFSS simulation.

#### Explanation of S-Parameters:

- dB(S(1,1)): Reflects the input signal loss at Port 1.

- dB(S(2,1)): Represents Near-End Crosstalk (NEXT)—how much signal at Port 1 couples into Port 2.

- dB(S(1,2)): Represents Far-End Crosstalk (FEXT)—how much signal at Port 2 couples into Port 1.

- dB(S(2,2)): Reflects the input signal loss at Port 2.

#### Observations:

- The S-parameter magnitudes (in dB) exhibit periodic variations due to signal reflections and interactions.

- NEXT (S21) and FEXT (S12) show significant coupling effects, especially at higher frequencies (e.g., around 15–20 GHz).

- S11​ and S22​ remain relatively low, indicating good signal retention at the input ports.

The extracted S21​ and S12​ values are later used in the Python simulation to simulate NEXT and FEXT signals.

3. Python Code Explanation

The Python code simulates Near-End Crosstalk (NEXT) and Far-End Crosstalk (FEXT) using the S-parameters extracted from HFSS.

Steps in the Code:

- CSV File Reading and Cleaning:

  - The program loads the CSV file containing S-parameters (dB(S(2,1)), dB(S(1,2)), etc.).

  - Column names are cleaned to remove extra spaces and special characters for ease of access.

- S-Parameter Conversion:

  - S21​ (NEXT) and S12​ (FEXT) are provided in dB scale.

  - The values are converted to linear scale using the formula:
             Linear = 10^dB/20

- Time Domain Simulation:

  - A unit step function is defined as the input signal.

  - Interpolation of S21​ and S12​ values ensures alignment with the simulation time range.

- Crosstalk Functions:

  - NEXT Simulation:

    - NEXT is proportional to the derivative of the input signal.

    - The Python function multiplies the input signal with the interpolated S21S_{21}S21​ coefficient.

  - FEXT Simulation:

    - FEXT is a delayed version of the input signal scaled by S12​.

    - A time delay is introduced using np.roll().

- Visualization:

  - Plots the Input Signal, NEXT, and FEXT to show the signal behaviors clearly.

Code:

import numpy as np

import pandas as pd

import matplotlib.pyplot as plt

# Load the CSV file

file_path = r"C:\Users\dagit\OneDrive\Desktop\Fall 2024\EE 486,486 Signal and Power Integrity\HFSS Project\Project 2\S Parameter Plot 1.csv"

try:

data = pd.read_csv(file_path)

except FileNotFoundError:

print(f"Error: File not found at '{file_path}'. Check the path and try again.")

exit()

# Clean column names by removing spaces, brackets, and special characters

data.columns = data.columns.str.strip().str.replace(r"[^\w]", "", regex=True)

# Debugging: Print cleaned column names

print("Cleaned Column Names:", data.columns)

# Extract relevant columns

try:

frequency = data['FreqGHz'] * 1e9  # Convert GHz to Hz

S21_dB = data['dBS21']  # NEXT data in dB

S12_dB = data['dBS12']  # FEXT data in dB

except KeyError as e:

print(f"Error: {e}. Check your file for column names.")

exit()

# Convert S-parameters from dB to linear scale

S21_linear = 10 ** (S21_dB / 20)  # NEXT coefficient

S12_linear = 10 ** (S12_dB / 20)  # FEXT coefficient

# Time settings for simulation

time = np.linspace(0, 2e-9, 1000)  # Time range (0 to 2 ns)

unit_step = np.heaviside(time, 1)  # Unit step input signal

# Interpolate S-parameter coefficients for the time domain

NEXT_coeff = np.interp(time, np.linspace(0, max(time), len(S21_linear)), S21_linear)

FEXT_coeff = np.interp(time, np.linspace(0, max(time), len(S12_linear)), S12_linear)

# Crosstalk transfer functions

def NEXT_signal(input_signal, coeff):

"""Simulate Near-End Crosstalk (NEXT)."""

next_signal = coeff * np.diff(input_signal, prepend=0)

return next_signal

def FEXT_signal(input_signal, coeff, delay=50):

"""Simulate Far-End Crosstalk (FEXT)."""

delayed_signal = np.roll(input_signal, delay)

delayed_signal[:delay] = 0  # Zero out initial delay

fext_signal = coeff * delayed_signal

return fext_signal

# Simulate the signals

next_output = NEXT_signal(unit_step, NEXT_coeff)

fext_output = FEXT_signal(unit_step, FEXT_coeff, delay=50)

# Plot results

plt.figure(figsize=(10, 8))

# Input Signal

plt.subplot(3, 1, 1)

plt.plot(time, unit_step, color='blue')

plt.title("Input Signal (Unit Step)")

plt.xlabel("Time (s)")

plt.ylabel("Amplitude")

plt.grid()

# Near-End Crosstalk (NEXT)

plt.subplot(3, 1, 2)

plt.plot(time, next_output, color='red')

plt.title("Near-End Crosstalk (NEXT)")

plt.xlabel("Time (s)")

plt.ylabel("Amplitude")

plt.grid()

# Far-End Crosstalk (FEXT)

plt.subplot(3, 1, 3)

plt.plot(time, fext_output, color='green')

plt.title("Far-End Crosstalk (FEXT)")

plt.xlabel("Time (s)")

plt.ylabel("Amplitude")

plt.grid()

plt.tight_layout()

plt.show()

4. Simulated Results

The third image displays the simulation results for the input signal, NEXT, and FEXT.

Plot Breakdown:

- Input Signal (Top Plot):

  - The input is a unit step function, which jumps from 0 to 1 at time t=0.

  - This represents the driving signal injected into the microstrip line.

- Near-End Crosstalk (NEXT) (Middle Plot):

  - The NEXT signal shows a sharp pulse at t=0, which corresponds to the derivative of the unit step signal.

  - The amplitude of the NEXT signal is scaled by the S21​ coefficient (NEXT coupling strength).

- Far-End Crosstalk (FEXT) (Bottom Plot):

  - The FEXT signal shows a delayed and weaker version of the input signal.

  - The delay occurs because FEXT happens at the far end of the line and takes time to propagate.

  - The signal's amplitude is scaled by the S12​ coefficient (FEXT coupling strength).

### 5. Conclusion

- HFSS Design:

  - The HFSS model successfully simulates the microstrip line configuration and calculates the S-parameters.

- S-Parameter Results:

  - S21​ (NEXT) and S12​ (FEXT) demonstrate the coupling effects between two closely spaced lines.

- Python Simulation:

  - The Python code utilizes the extracted S-parameters to model NEXT and FEXT in the time domain for a unit step input signal.

  - The results validate the crosstalk behavior, showing both the derivative nature of NEXT and the delayed nature of FEXT.

- Visualization:

  - The final plot provides a clear representation of the input signal and its coupled effects (NEXT and FEXT).